SIMATS ENGINEERING

ASSESSMENT TOOL 1 Experiments

COURSE NAME: Computer Networks  	 	COURSE CODE: 	CSA07

COURSE OUTCOME COVERED

CO3: Analyze the performance of core network protocols such as TCP, UDP, HTTP, and

DNS under varying conditions

Assessment Tool Weightage: 40%

SOFTWARE: CISCO PACKET TRACER, WIRESHARK

EXPERIMENTS

QUESTIONS:4 	 	 	 	 	 	                Total Marks:40

- Design a simple LAN consisting of two PCs connected through a switch using Cisco Packet Tracer. Configure appropriate IP addresses for both hosts and verify connectivity using the ping command. Simultaneously capture the packet exchange using Wireshark. Identify the Ethernet frame fields such as Source MAC Address, Destination MAC Address, EtherType, and Frame Check Sequence (FCS). Explain how the Data Link layer uses MAC addressing to ensure successful frame delivery within the local network.

- Create a network topology consisting of two LANs connected through a router in Cisco Packet Tracer. Configure IPv4 addressing, subnet masks, and default gateways for all devices, and verify end-to-end communication using the ping command. Capture the ICMP packets using Wireshark and analyze the IPv4 header fields, including Source IP Address, Destination IP Address, Time-ToLive (TTL), Protocol Number, and Header Length. Explain the role of the Network layer in forwarding packets between different networks.

- Construct a local area network with multiple hosts connected through a switch in Cisco Packet Tracer. Clear the ARP cache on one of the hosts and initiate communication with another host using the ping command. Capture the communication in Wireshark and identify the ARP Request and ARP Reply packets. Analyze the sender and target MAC/IP addresses and explain how the Address Resolution Protocol resolves logical IP addresses into physical MAC addresses before data transmission.

- Develop a simple network consisting of two PCs connected through a router in Cisco Packet Tracer. Configure the required IP addresses and test connectivity using the ping command. Capture the ICMP traffic using Wireshark and identify the Echo Request and Echo Reply messages. Examine important ICMP fields such as Type, Code, Identifier, and Sequence Number, and calculate the Round Trip Time (RTT). Discuss how ICMP assists in network troubleshooting and connectivity verification.

- Create a client-server network in Cisco Packet Tracer by configuring a web server and a client PC. Enable the HTTP service on the server and access the hosted webpage from the client using a web browser. Capture the communication in Wireshark and identify the TCP three-way handshake, followed by the HTTP GET request and HTTP response messages. Analyze the application-layer communication and explain how TCP provides reliable delivery for HTTP traffic through connection establishment, acknowledgments, and ordered data transfer.

RUBRICS FOR CALCULATION

| Criteria | Weightag e | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understandin g of Concepts | 25 | Demonstrates thorough understanding and effectively integrates multiple concepts/technologi es | Good understanding with proper integration of most concepts | Basic understanding ; limited or partial integration | Poor understanding ; unable to integrate concepts |
| Experimental Design | 30 | Well-planned, systematic implementation with optimal solution | Correct implementati on with minor issues | Basic implementati on with errors or inefficiencies | Incorrect or incomplete implementati on |
| Use of Tools | 20 | Effective and advanced use of tools/technologies with proper configuration | Appropriate use of tools with correct execution | Limited or inconsistent use of tools | Incorrect or minimal use of tools |
| Analysis of Results | 15 | Insightful analysis with accurate interpretation and validation of results | Good analysis with reasonable interpretation | Basic analysis with limited insights | Poor or incorrect analysis of results |
| Documentati on | 10 | Clear, wellstructured documentation with diagrams, code, and results | Organized documentatio n with necessary details | Basic documentatio n with missing details | Poor or incomplete documentatio n |

4.

Aim:

To Develop a simple network consisting of two PCs connected through a router in Cisco Packet Tracer. Configure the required IP addresses and test connectivity using the ping command. Capture the ICMP traffic using Wireshark and identify the Echo Request and Echo Reply messages. Examine important ICMP fields such as Type, Code, Identifier, and Sequence Number, and calculate the Round Trip Time (RTT). Discuss how ICMP assists in network troubleshooting and connectivity verification

Procedure:

  Open Cisco Packet Tracer and create a network with two PCs and one router.

  Connect PC1 to Router and Router to PC2 using Copper Straight-Through cables.

  Configure PC1 with:

- IP Address: 192.168.1.2

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

  Configure PC2 with:

- IP Address: 192.168.2.2

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.2.1

  Configure the router interfaces:

- G0/0 → 192.168.1.1

- G0/1 → 192.168.2.1

- Enable both interfaces using the no shutdown command.

  Verify the router interfaces using the command:
show ip interface brief

  Open PC1 → Desktop → Command Prompt and use the command:
ping 192.168.2.2

  Check whether Echo Request and Echo Reply messages are received successfully.

  Open Wireshark and start capturing network packets while performing the ping operation.

  Apply the filter icmp in Wireshark to display only ICMP packets.

  Select an Echo Request packet and note the Type, Code, Identifier, and Sequence Number fields.

  Select the corresponding Echo Reply packet and compare its ICMP fields with the request.

  Calculate the Round Trip Time (RTT) by subtracting the Echo Request timestamp from the Echo Reply timestamp.

  Analyze the ICMP packets to verify network connectivity and understand how ICMP helps in troubleshooting communication problems.

  Record the obtained ping results, ICMP fields, and RTT in the observation section

Implementation:

Step 1:

Explanation:

  - Network Topology

The first step creates the basic network.

- Two PCs (PC0 and PC1) are connected to one router (Router0).

- PC0 is connected to FastEthernet0/0 of the router.

- PC1 is connected to FastEthernet0/1 of the router.

- Two different networks are used:

  - PC0 network: 192.168.1.0/24

  - PC1 network: 192.168.2.0/24

IP Addresses

| Device | IP Address |
| --- | --- |
| PC0 | 192.168.1.2 |
| Router Fa0/0 | 192.168.1.1 |
| Router Fa0/1 | 192.168.2.1 |
| PC1 | 192.168.2.2 |

Step 2:

Explanation:

. PC0 IP Configuration

In this step, the IP address of PC0 is configured.

Go to:

PC0 → Desktop → IP Configuration

Enter:

- IP Address: 192.168.1.2

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

PC0 belongs to the 192.168.1.0 network. The router's 192.168.1.1 address is used as the gateway to reach other networks.

Step 3:

Explanatiom:

PC1 IP Configuration

In this step, the IP address of PC1 is configured.

Go to:

PC1 → Desktop → IP Configuration

Enter:

- IP Address: 192.168.2.2

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.2.1

PC1 belongs to the 192.168.2.0 network. The router's 192.168.2.1 address acts as the gateway for PC1.

Step 4:

Explanation:

Router Configuration (CLI)

In this step, the two router interfaces are configured.

Open:

Router0 → CLI

- enable → enters privileged mode.

- configure terminal → enters configuration mode.

- interface fastethernet 0/0 → selects the interface connected to PC0.

- ip address → assigns an IP address to the interface.

- no shutdown → activates the interface.

- interface fastethernet 0/1 → selects the interface connected to PC1.

The router now has an interface in each network and can route packets between them.

Step 5:

Explanation:

Verify Router Interfaces

The command:

show ip interface brief

is used to check the router interfaces.

The up/up status means:

- The interface is enabled.

- The physical connection is active.

- The router can communicate through that interface.

Therefore, both router connections are working correctly.

Step 6:

Explanation:

ing from PC0 to Router

Open:

PC0 → Desktop → Command Prompt

Type:

ping 192.168.1.1

this test checks whether PC0 can communicate with its default gateway, which is the router.

Since all four packets are received:

Packets Sent = 4

Received = 4

Lost = 0

the connection between PC0 and Router0 is successful.

Step 7:

Explanation:

Ping from PC0 to PC1

Now PC0 is used to test communication with PC1.

In:

PC0 → Desktop → Command Prompt

type:

ping 192.168.2.2

The image shows:

Reply from 192.168.2.2

with 4 packets received and 0% packet loss.

This confirms that PC0 can communicate with PC1 through the router.

Step 8:

Explanation:

Simulation Mode – ICMP Packet Flow

This is an important step for your experiment.

In Cisco Packet Tracer, click:

Simulation at the bottom-right.

Then perform the ping again.

You will see packet envelopes moving:

PC0 → Router0 → PC1

and then returning:

PC1 → Router0 → PC0

The purple envelope represents:

ICMP Echo Request

The white envelope represents:

ICMP Echo Reply

Explanation

When PC0 sends a ping:

PC0 sends an ICMP Echo Request → Router forwards it → PC1 receives it.

PC1 then responds:

PC1 sends an ICMP Echo Reply → Router forwards it → PC0 receives it.

This visually demonstrates how ICMP travels through the network.

Step 9:

Explanation:

Event List – Simulation Mode

The Event List shows the ICMP packets generated during the ping.

Your image shows entries such as:

| Device | Type | Information |
| --- | --- | --- |
| PC0 | ICMP | Echo Request |
| Router0 | ICMP | Echo Request |
| PC1 | ICMP | Echo Request |
| PC1 | ICMP | Echo Reply |
| Router0 | ICMP | Echo Reply |
| PC0 | ICMP | Echo Reply |

This shows the complete journey of the ICMP packets.

Echo Request

PC0 → Router0 → PC1

Echo Reply

PC1 → Router0 → PC0

This confirms that the router successfully forwards the ICMP packets between the two networks.

RESULT:

Thus, the network was successfully configured and connectivity between PC0 and PC1 was verified using ICMP ping. The Simulation Mode displayed the ICMP Echo Request (Type 8) and Echo Reply (Type 0), demonstrating how ICMP is used for connectivity verification and network troubleshooting.

5.

AIM:

To Create a client-server network in Cisco Packet Tracer by configuring a web server and a client PC. Enable the HTTP service on the server and access the hosted webpage from the client using a web browser. Capture the communication in Wireshark and identify the TCP three-way handshake, followed by the HTTP GET request and HTTP response messages. Analyze the application-layer communication and explain how TCP provides reliable delivery for HTTP traffic through connection establishment, acknowledgments, and ordered data transfer.

PROCEDURE:

- Open Cisco Packet Tracer and add one PC, one Switch, and one Server.

- Connect the PC and Server to the Switch using Copper Straight-Through cables.

- Configure the PC with IP address 192.168.1.2 and subnet mask 255.255.255.0.

- Configure the Server with IP address 192.168.1.10 and subnet mask 255.255.255.0.

- Open Server → Services → HTTP and turn HTTP ON.

- From the PC, open Command Prompt and use ping 192.168.1.10 to verify connectivity.

- Open Web Browser on the PC and enter http://192.168.1.10 to access the webpage.

- Start Wireshark and capture the network communication.

- Apply the tcp or http filter to identify the packets.

- Identify the TCP three-way handshake: SYN, SYN-ACK, and ACK.

- Identify the HTTP GET request sent by the client and the HTTP 200 OK response from the server.

- Observe TCP acknowledgments and sequence numbers to understand reliable and ordered data delivery.

- Record the observations and conclude that TCP provides reliable delivery for HTTP communication.

IMPLEMENTATION

STEP 1:

EXPLANATION:

Create the Network Topology

Open Cisco Packet Tracer.

Add:

- 1 PC

- 1 Server

- 1 Switch

Connect them using Copper Straight-Through cables:

PC0 -------- Switch -------- Server0

Explanation

The PC acts as the client, the Server acts as the web server, and the Switch connects both devices within the local network.

STEP 2:

EXPLANATION:

Configure the Client PC

Click:

PC0 → Desktop → IP Configuration

Select Static.

Enter:

- IP Address: 192.168.1.2

- Subnet Mask: 255.255.255.0

You can leave the gateway empty because both devices are in the same network.

Explanation

The client PC is assigned the IP address 192.168.1.2.

STEP 3:

Configure the Web Server

Click:

Server0 → Desktop → IP Configuration

Enter:

- IP Address: 192.168.1.10

- Subnet Mask: 255.255.255.0

Explanation

The server is assigned 192.168.1.10. Since both the PC and server are in the 192.168.1.0/24 network, they can communicate directly through the switch.

STEP 4:

Enable HTTP Service

Click:

Server0 → Services → HTTP

You will see the HTTP service settings.

Make sure:

HTTP = ON

You can keep the default webpage.

Explanation

HTTP is the protocol used to transfer webpages between a web server and a client.

The server now acts as an HTTP web server.

STEP 5:

Test the Connection Using Ping

Go to:

PC0 → Desktop → Command Prompt

Type:

ping 192.168.1.10

You should get:

Reply from 192.168.1.10

Explanation

This confirms that the client can communicate with the web server.

STEP 6:

Open the Web Browser

On PC0:

Desktop → Web Browser

In the URL/address box, enter:

http://192.168.1.10

Click Go.

You should see the webpage hosted by Server0.

Explanation

The PC sends an HTTP request to the server.

The server processes the request and sends the webpage back to the PC.

The communication is:

PC0 → HTTP Request → Server0

PC0 ← HTTP Response ← Server0

STEP 7:

Capture the Communication

For actual Wireshark analysis, start Wireshark on the real/virtual network interface carrying the HTTP traffic.

Important: Packet Tracer's simulated packets are not normally captured directly by Wireshark. You can use Packet Tracer's Simulation Mode to observe the packet flow, while Wireshark is used with an actual/virtual network interface for packet-level capture.

Start the Wireshark capture and then access:

http://192.168.1.10

from the client.

STEP 8:

EXPLANATION:

Filter TCP Packets

In Wireshark, use:

tcp

You will see TCP packets.

For HTTP traffic, you can also use:

http

Explanation

HTTP uses TCP for reliable transport.

Usually, the TCP connection is established before the HTTP GET request is sent.

STEP 9:

EXPLANATION:

Identify TCP Three-Way Handshake

Look for these three packets:

1. SYN

Client → Server

SYN

The client requests a TCP connection.

2. SYN-ACK

Server → Client

SYN + ACK

The server accepts the connection request and acknowledges it.

3. ACK

Client → Server

ACK

The client acknowledges the server's response.

Therefore:

Client                         Server

|                              |

| -------- SYN -------------> |

| <------- SYN + ACK -------- |

| -------- ACK -------------> |

|                              |

|    TCP Connection Ready      |

Explanation

This is called the TCP three-way handshake.

It establishes a reliable TCP connection before HTTP data is exchanged.

STEP 10:

Identify HTTP GET Request

After the handshake, look for an HTTP packet containing:

GET / HTTP/1.1

The direction will be:

Client → Server

Explanation

The GET request asks the server to send the webpage.

For example:

Client → Server

HTTP GET

The client is basically requesting:

"Send me the webpage."

STEP 11:

Identify HTTP Response

After the GET request, look for the server's HTTP response.

You may see something such as:

HTTP/1.1 200 OK

Direction:

Server → Client

Explanation

200 OK indicates that the server successfully processed the request and returned the requested resource.

STEP 12:

Observe TCP ACKs

In Wireshark, observe the TCP packets containing ACK.

TCP acknowledges received data.

Explanation

When the server sends data, the client acknowledges its successful reception.

Similarly, data sent by the client is acknowledged by the server.

This helps TCP provide reliable delivery.

STEP 13

:

Application Layer Analysis

The communication involves several layers:

| Layer | Protocol | Function |
| --- | --- | --- |
| Application | HTTP | Transfers webpage |
| Transport | TCP | Reliable delivery |
| Internet | IP | Addressing and routing |
| Network Access | Ethernet | Transfers frames |

Main communication

HTTP GET

↓

TCP Connection

↓

IP

↓

Ethernet

STEP  14:

| Communication | Direction | Purpose |
| --- | --- | --- |

| SYN | Client → Server | Request TCP connection |
| --- | --- | --- |

| SYN-ACK | Server → Client | Accept and acknowledge |
| --- | --- | --- |

| ACK | Client → Server | Confirm connection |
| --- | --- | --- |

| HTTP GET | Client → Server | Request webpage |
| --- | --- | --- |

| HTTP Response | Server → Client | Send webpage |
| --- | --- | --- |

| TCP ACK | Both directions | Acknowledge received data |
| --- | --- | --- |

RESULT:

The client-server network was successfully created in Cisco Packet Tracer. The HTTP service was enabled on the web server, and the webpage was successfully accessed from the client PC. The TCP three-way handshake consisting of SYN, SYN-ACK, and ACK was identified, followed by the HTTP GET request and HTTP response. TCP provides reliable HTTP communication through connection establishment, acknowledgments, sequence numbers, retransmission, and ordered data delivery.